Codes for older Panasonic and Technics    Carrier: 32kHz            +--------+      +.. Header:   |        |      |         ..+        +------+               8T     6T                +--+    +--+ 1 is coded:  |  |    |  |            ..+  +----+  +..               2T  6T  2T               +--+  +--+ 0 is coded:  |  |  |  |            ..+  +--+  +..               2T 2T 2T  T=420 us to approx 424 us (US of A and Canada) T=454 us to approx 460 us (Europe and others)   Remote control transmitter data code (MN6030)  Head pulse  Custom code  Data code  Inverse custom code  Inverse data code (8T high,   (5 bits)     (6 bits)   (1<==>0)             (1<==>0)  8T low  Followed by 90T low (for sync) = 41130us (Europe) = 37980us (USA)  Old Matsushita infostring: (use ASC-> to recall the string) OLDMATSU (Europe)  To use the codes recall infosting and put the number on level 1 of  stack. Example to play a panasonic video:  \<<   OLDMATSU		@ get info string   #142h			@ get the data   TRANSMIT		@ transmit \>>  ****************************** Panasonic  ------------------------------ Video  vtr		402 timer rec	6C2 1		202 2		222 3		242 4		262 5		282 6		2A2 7		2C2 8		2E2 9		302 10/0		322 -/--		482 Ch +		422 Ch -		462 play		142 stop		002 rew		042 ff		062 pause		0C2 rec		102 still adv	182 slow		1E2 prog		583 next		743 vps		0C3  -------------------------- TV  On/Off		400 1		200 2		220 3		240 4		260 5		280 6		2A0 7		2C0 8		2E0 9		300 0		320 -/--		480 C		420 Ch +		440 Ch -		460 Mute		160 TV/AV		540 N		0E0 <-|-> -		580 <-|-> +		560 Color -		1C0 Color +		180 Contrast - 	5C0 Contrast +	500 Brightness -	1E0 Brightness +	0C0 Treble -	380 Treble +	4E0 Bass -		3A0 Bass +		5E0 TV		740 TEXT		760 Alarm		3E0 Mix		780 Reveal		380 List Store	4E0 Index		5E0 Hold		680 Time text	6E0 Display cancel	6A0 F/T/B		720 List/F. Text	3A0 Red		340 Green		360 Yellow		6C0 Blue		7C0  ********************************* Technics  --------------------------------- Amp  on/off		7E7 Muting		79D Vol +		7FB Vol -		70D Eq on/flat	609 Preset		649  --------------------------------- CD  Play/Pause	14C Stop		00C Rew		04C FF		06C Repeat		0EC Time mode	52C Level +		48C Level -		4AC Memory		54C Clear		16C 1		20C 2		22C 3		24C 4		26C 5		28C 6		2AC 7		2CC 8		2EC 9		30C 0		32C	  --------------------------------- Tape  Deck a/b	089 Play		149 RewPlay		129 Stop		009 Rew		069 FF		049 Pause		109  --------------------------------- Tuner  1		209 2		229 3		249 4		269 5		289 6		2A9 7		2C9 8		2E9 9		309 0		329  --------------------------------- Phono  Start		5FB Stop		3FB  ---------------------------------