Codes for Sony remotecontrol emulation

The code is 12 bit long (15 bit for videocameras).

Carrier:	40kHz

+--------+

Header:      |        |

..+        +--

4T    T

+----+

1 is coded:  |    |

..+    +--

2T  T

+--+

0 is coded:  |  |

..+  +--

T  T

T = 550us approx

Space between data:	25ms

data: hhhhxxxxxxyyyyyy

^          ^

MSB        LSB

xxxxxx command yyyyyy

address ( the number of the apparatus )

tape deck a	# 20

tape deck b	# 1C

dat		# 38

cd		# 22

Sony infostring: (use ASC-> to recall the string)

%%HP: T(1);

@ SONY "C2A2052000C4D00352D00511D005C00020020202330AB7"

To use the codes recall infosting and put the number on level 1 of  stack.

Example to play a sony cd:

\<<   SONY			@

get info string   #8B2h			@

get the data   TRANSMIT		@

transmit

\>>  ***************************

tape	deck a	deck b	dat

---------------------------

play	832	71A	E3A

rew	833	71B	E3B

ff	834	71C	E3C

rec	835	71D	E3E

rewplay	837	720	---

stop	838	718	E38

pause	839	719	E39

rewrec	83A	712	---

vtr	vtr 1	vtr 2	vtr 3

-----------------------------

ch+	110	390	590

ch-	111	391	591

power	115	395	595

stop	118	398	598

pause	119	399	599

play	11A	39A	59A

rew	11B	39B	59B

ff	11C	39C	59C

rec	11D	39D	59D

ant	12A	3AA	5AA

cd ----------------------------

880	1

881	2

882	3

883	4

884	5

885	6

886	7

888	8

889	9

88A

88B

88C

88D

88E

88F	Clear

890	17

891	18

892	Line Out Volume Up

893	Line Out Volume Down

894

895

896	Open/Close

897	19

898	Custom Index

899

89A

89B

89C

89D	Continue

89E

89F	Program

8A0	10

8A1	11

8A2	12

8A3	13

8A4	14

8A5	15

8A6	16

8A7	+10 (untested)

8A8	Time

8A9	20

8AA	Repeat A<->B

8AB

8AC	Repeat 1/All

8AD

8AE

8AF

8B0	AMS Backward

8B1	AMS Forward

8B2	Play

8B3	Fast Audible Search Backward

8B4	Fast Audible Search Forward

8B5	Shuffle

8B6	Index Backward

8B7	Index Forward

8B8	Stop

8B9	Pause

8BA	Slow Audible Search Backward

8BB	Slow Audible Search Forward

8BC

8BD

8BE

8BF

8C0     Select Disc #1

8C1     Select Disc #2

8C2     Select Disc #3

8C3     Select Disc #4

8C4     Select Disc #5

amp ----------------------------

power		815

vol-		813

vol+		812

rear lvl-	643

rear lvl+	642

dbfb		631

muting		814

cd		825

tuner		821

phono		820

tape		823

dat		846

video 1		822

video 2		81E

video 3		842

video 4		843

dolby		639

hall		63A

simulated	63B

off		638

delay time	644

phono ---------------------------

start		828

stop		829

tuner ---------------------------

shift		63B

pre+		690

pre-		691

tv ---------------------------

power		095

vol+		092

vol-		093

ch+		090

ch-		091

tv/video	0A5

1		080

2		081

3		082

4		083

5		084

6		085

7		086

8		087

9		088

0		089

enter		08B

ldp ---------------------------

power		315

play		31A

stop		318

skip rew	335

skip ff		334

rew		31B

ff		31C

videocamera ---------------------------

Play		39A

Stop		398

Rew		39B

FF		39C

Pause		399

Data screen	3DA

Tele		6C9A

Wide		6C9B

Start/Stop	5C99

Tele Wide and Start/Stop uses 15 bits and requires SONYCAM as infostring in order to work. The other codes the normal SONY infostring.