- Thought MIT profile probe was a significant achievement

- agrees that there are alternatives to prodding

- not too thrilled about prodders

- prodding not the main issue – excavation

- places with no detection problem

- most groups still use bayonets

- places like Afghanistan have basically hard concrete like ground – rocks, mud and prodding hard

- When he was in Afgan, 85 accidental set offs of PMN

- Angle – Gasser thinks that there is an issue with it – but it’s his personal one

- More problem is force

- Deminers do not realize that they are pushing too hard

  - in wetlands – mines degrade after 10 years

  - hitting edge of mine

  - Rusting of mines

  - In Mozambique – only 2 per acre – deminers not finding anything for awhile and getting sloppy

- really need to change SOP – deminers will not stop pushing too hard until they are fined/not paid for the day that they are discovered putting too hard

- but need to make sure that they are still pushing into the ground

- therefore device needs to change color – or do something that has to be reset by the supervisor

-a bit of a digression-

- demining is really not all technical –

- must go by a systems approach – wide religious/cultural to engineering

- SOP needs to be different for each place

- SOP needs to be updated

- Would have liked to have a probe in the field – if in the field, there can be no loose part that would come off in case of a blast

- Angle tool

  - Gasser thought of a tilt switch (like the mecury)

  - The end of the probe is a cone/funneled shaped – wire on bottom with conductive liquid

  - Therefore, you can rotate the probe 360 deg but if you tilt the angle more that 30 deg, the conductive liquid looses contact from the wire and the tools malfunction

- Gasser was very focused on electronics – EE person – liked the chip, battery, bluetooth, radio link…

- Chips that can log angle and force for a few dollars

- His results need to be repeated – with force and angle should be looked into

- What he would do next if he had funding –

  - Rotating tip product

  - Vibration/Ultrasound

  - A prodder that vibrates at the resonant freq of gravel – gravel would be pushed away (is it worth complexity and cost?)

- should never distract deminers – senses are important (so have to think twice about sound, vibrations…)

- smart probe doesn’t help – not 100% able to tell what you what you have found so you have to excavate anyway

- for the probe, if several thousand are ordered, cost could be $10

- force sensing – strain gauges – micro switch, optical switch

-somehow metal extraction came around-

- good!

- Should look into

- GICHD Geneva manual – Tim Lardner