Documentation for the program "calamp" CJL, September 25 1996

-------------------------------------------------------------

COMMAND NAME:   calamp

SYNOPSIS:  Applies calibration data to A-file amplitudes

SYNTAX:  calamp [-c caltime] [-m msglev] [-o outfile] [-s] afilename calfilename

-c allows control over required proximity of cal time

-m controls verbosity

-o overrides stdout default for output A-file

-s forces program to use cal only on same source

-t forces program to ignore Tant information

afilename is the uncalibrated A-file input

calfilename is file containing all cal info

OPTION FLAGS:	-c caltime

For each VLBI scan, calamp will accept cals within

caltime seconds of the start and end of the scan

(more precisely, any cal after start-caltime and

before end+caltime), and will simply average the

cal values it finds.  The default is 600 seconds.

-m msglev

Verbosity control parameter, ranging from -3 to +3,

with lower numbers being more verbose.  The default

value is 1.

-o outfile

If specified, this flag directs the A-file output

of calamp to the named file.  If omitted, calamp

writes its output to stdout.

-s

Normally, calamp uses all cal scans within the allowed

time range, issuing only a warning at message level 1

when the cal source does not match the VLBI source

(message level -1 gives more details).  This flag turns

the non-matching source condition into a non-valid

cal, which then gets completely ignored (with no

messages).

-t

Normally, when a 6-token line is encountered in the

cal file, calamp performs a Ta/source flux calculation.

This flag suppresses this behaviour, and forces a

Tsys/gain curve calculation.

ARGUMENTS:	afilename

This is normally the final output of the fringe

searching software, with amplitudes in units of

10**-4 correlation coefficient

calfilename

This is a file containing calibration information

feels like	for all stations.  It is normally the output of the

RCal program, written by John Ball, modified by hand

editing in most cases.

ENVIRONMENT:	None.

DESCRIPTION:

Description deferred until someone other than the authors finally finishes it.