* Current syntax for fourfit control file  - last modified 93.12.28

max_parity 0.001                       * global commands come first

ref_freq  8213.15

x_crc       discard

x_slip_sync keep

y_slip_sync discard

if station L and f_group X

freqs a b c d e f g h

pc_phases abcdefgh 5 -11 12 38 -56 13.2 11 -29

pc_mode ap_by_ap

if station L and f_group S

pc_phases ijkmn 4.5 -78 39 +12 0

pc_mode normal

if (station V or baseline KT) and source 3C279

sb_win -0.5 0.5    mb_win 0.02 0.02  dr_win -1.0E-6 0.5E-6

else

sb_win 0.0 0.0     mb_win 0.02 0.02  dr_win -1.0E-6 0.5E-6

if scan 288:210210

sb_win .37 .37

if scan > 289:132510.

skip

if baseline K?

switched scan_start

period 30

gates abcfgh  0 30  0 10  15 25     0 10  15 25  0 30

* End of sample control file

SELECTOR KEYWORDS       VALUES

station            1 character

baseline           2 characters

source             string

scan               time-value (special format)

f_group            1 character

SYNTACTIC KEYWORDS

if

else   (NYI)

and

or

not

()     (NYI)

<>     (NYI)

?

ACTION KEYWORDS         VALUES

max_parity         float

x_slip_sync        `keep' or `discard'

y_slip_sync          "        "

x_crc              `keep', `discard', or integer

y_crc                "      "           "

freqs              n chars

index              n ints

pc_phases          n char string, followed by n floats

lsb_offset            "        "      "     "

ref_freq           float

pc_mode            `normal', `ap_by_ap', or `manual'

sb_win             2 floats

mb_win             "   "

dr_win             "   "

start              time-value

stop                "     "

skip               `true' or `false'(NYI) - for now use 1 or 0

switched           `scan_start' or `each_minute'

period             int

gates              n char string, followed by 2n floats

SPECIAL KEYWORD VALUES

?         wild card character

keep      32767

discard       0

FORMATS

Time-values:

(NYI)Absolute times are in the format ddd:hhmmss.mmm, where the colon

or decimal point can "anchor" surrounding numbers. If both

are absent, then there is an implied decimal to the right of

the last number. If both are present, there must be 6 intervening

characters (dddhhmmss). Examples:

field           ddd hh mm ss mmm

-----           --- -- -- -- ---

208:           208 00 00 00 000

208            000 00 02 08 000

:1432           000 14 32 00 000

:8123456.            ** invalid **

Relative times are depicted by a leading + or -. The rest of the

format is the same as for absolute times.

As of 93.5.21 a cruder method of entering times is all that is supported.

Arguments of start and stop are integers with an optional minus sign. A

positive integer is interpreted as an absolute time in seconds past the

hour (of the scan start time). When a minus sign precedes the start time

it is considered to be a time relative to, and later than, the

scheduled scan start. Similarly, a negative stop time precedes the

scheduled scan stop time, by the indicated number of seconds.

GENERAL GUIDELINES

1) White space is ignored; i.e., multiple spaces and line feeds all

collapse to a single space.

2) Multiple commands per line are fine.

3) Comments: anything from an asterisk through the end of the line

is ignored.

4) Nested ifs are not allowed (or necessary). Nested parentheses in

an if condition are fine.

5) Wildcard "?" matches any single character for f_group, station, or

baseline, any string (of up to 8 characters) for source, and any

time-value for scan.

6) Phase cal is normally treated station by station. Only freqs that are

chosen for both stations in a baseline are present in the fit.