Documentation for the program "fourfit", CJL/RJC 26 November 1993

-----------------------------------------------------------------

COMMAND NAME:   fourfit

SYNOPSIS:  Performs fringe searching for continuum MkIII/MkIV data

SYNTAX:  fourfit [-a] [-b BB:F] [-c controlfile] [-d display device] [-f value]

[-m value] [-n value] [-p] [-r afile] [-s naps] [-tux] [-P polar_pair]

[-T trefoffs] [-X] data file list

[set <control file syntax statements>]

Where all arguments except the data file list are optional.

The [-r afile] option replaces the data file list, however.

The "set" argument and the commands which follow it must

come last.  All option flags must appear before the data file

list.  Option flags can come in any order.

Here are two examples of command-line invocations of fourfit, with

an explanation of what they do:

fourfit -txas -m 1 -c control 018-234505 set mb_win -0.0034 .004 freqs a b

Test mode, xwindow display, accounting switched on, cross power

spectrum plot switched on, moderately verbose, use control file

named "control" in current working directory, process all data in

scan directory 018-234505, override multiband delay search

window and select channels 'a' and 'b' only.

fourfit -r refr_list -c control -d hardcopy -b AT:S

Process all data referenced by type 2 lines in the A-file named

"refr_list", use control file "control", print the fringe plot

on the default printer, process only baseline AT frequency

subgroup S.

OPTION FLAGS:

-a

If specified, this option switches on accounting

of CPU time and wall-clock time used in the various

parts of fourfit.  When the program finishes, it

produces a summary of these timing statistics.

-b BB:F

Allows the user to override the control file

with a specification of the baseline and/or

frequency group to be processed.  The syntax is

flexible.  0, 1 or 2 characters before the colon

refer to the baseline (one character is interpreted

as a station), and 0 or 1 character after the colon

is interpreted as the frequency subgroup.  You can

use the control file wildcard character '?' in

the baseline, but remember to protect it from the

C-shell either by escaping it with a backslash '\'

or enclosing the entire -b argument in single

quotes.  If you wish only to specify the baseline,

the colon may be omitted.  An error in the -b

flag argument causes the flag to be ignored, and

fourfit will continue execution.

-c controlfile

Specifies the file which contains parameters

to control the operation of the program.  If

absent, fourfit will use only the file pointed to

by the environment variable DEF_CONTROL, which

in turn defaults to $FF/cf_default as defined

in the $HOPS/setup.csh file.  Any parameters

set in a control file specified with the -c option

override the default file values.  A description

of the syntax of the control file, with an example,

can be found later in this document.

-d display_device

Upon completion of a fringe-fit, fourfit can

optionally display the results in graphical form.

At present, the only form of output is the

traditional fringe plot (somewhat modified from

the FRNGE version).  It can be presented in three

ways:  In an X-window popup, in a disk file, and

as hardcopy.  The corresponding values of the

string 'display_device' are "xwindow", "diskfile",

and "hardcopy".  For "xwindow", be sure to set

your DISPLAY environment variable appropriately.

The "diskfile" option is currently hardcoded to the

filename "temp.fplot" in the current working

directory.  The hardcopy option sends the plot to

a printer under control of the script

"/usr/local/bin/fplot_print".  At Haystack, this

currently uses the printer named "east".

-f first channel

overrides the default first channel (0) to facilitate

plotting when there are more than 16 channels (see -n)

-m value

This flag controls the verbosity of the program via

the integer argument "value", which ranges from 3

(virtually silent except for major errors) to -3

(incredibly verbose, of use only to the authors of

the program).  The default is 2.

-n number of channels

When channels are overridden (see also the -f flag)

this tells how many channels to put on the plot. Note

that neither -f nor -n affect the actual fringe fit,

just the plotting thereof.

-p

Causes a postscript plot to be displayed on an

X-window.

-r afile

Puts fourfit into "refringe" mode.  Fourfit refringing

is driven by an A-file input, which overrides any

correlator root files directly specified on the command

line (i.e. the latter are ignored when the -r flag

is specified).  The input A-file, of which there can

be only one, may contain root, corel or fringe lines,

but only the fringe lines are used to determine which

data to process, by baseline and frequency subgroup.

Obviously, the -r flag is inconsistent with the -u

flag, and specifying both is an error.  Note that for

afiles using HP-1000 (version 1) line formats, fourfit

has to pre-check the disk for the existence of the

type 2 files.  The data area is controlled by the

DATADIR environment variable.  It defaults to the

value of $CORDATA.

-s naps

This parameter controls how many AP's are merged

together into each plotting segment. Thus the number

of time points shown in the phase, amplitude, and

validity plots is so controlled. Additionally, the

ph/seg and amp/seg statistics are calculated based

upon the stated number of AP's in each segment.

-t

This flag places fourfit in test mode.  Everything

works as normal, except that the output file is not

written to disk, and the root file is not updated.

This is useful when experimenting with different

fringe-fitting strategies, in order to avoid cluttering

up the disk.

-u

Normally, fourfit processes all data consistent with

the data file list and the control information.  When

this flag is specified, fourfit will also check the

information in the type-2100 record of the root to

see if the data have already been processed by fourfit.

If so, the data in question are skipped.  The "u"

stands for update mode.

-x

Shorthand for -d xwindow.

-P pp

Controls polarization processing, where the 2 character

string pp is one of four cross-polarization

states {RR, LL, RL, or LR}

-T trefoffs

If this option is invoked, the fourfit reference

time will be calculated by taking the nominal scan

start time from the ovex file and adding trefoffs

(which is an integer # of seconds) to it.

-X

Forces fourfit to write cross-power spectra into

type 230 records. This option is typically used for

import into AIPS.

ARGUMENTS:	data file list

This mandatory argument or arguments tells fourfit

which data files to process.  The format of the data

file specification is the standard one for all MkIV

software.  You may specify individual filenames,

scan directories which contain data files,

experiment directories, or any combination of

these three.  In the latter two cases,

fourfit will descend the directory tree looking for

data files to add to its internal list of files to

process.  Only root files need be specified.  The

data files actually fringe-searched are determined

by the combination of the root files specified and the

restrictions imposed by the control file or control

parameter list (see below).  In the absence of

such restrictions, all data associated with the

specified root files are processed.

Beware of trying to specify too many

files or scan directories, as it is possible to

overflow the Unix argument list buffer on large

experiments.  In such cases, specify the experiment

directory instead.

set <control file syntax statements>

This command line argument is optional, and

is intended to permit rapid, temporary modification

of 'fourfit' behaviour without the need to edit the

control file.  The word "set" tells fourfit to expect

additional control information on the rest of the

command line.  The syntax of this control information

is identical to that of the control file (see

detailed description below).  The control file

parser will detect syntax errors and abort if you

do not follow the rules as laid down.  The control

information you specify after the "set" argument

on the command line applies to all data to be

processed, and overrides whatever the control file

itself specified for the parameters in question.

ENVIRONMENT:	DEF_CONTROL, DISPLAY, DATADIR

DESCRIPTION:

Fourfit is the functional analogue of FRNGE on the HP-1000 systems, and searches

the data represented by the root and corel files for fringes, writing the results

of the search to files of type fringe.  The emphasis in the design of the

program has been speed and flexibility, particularly with regard to future

enhancements.  Algorithmically, the program is closely modelled on FRNGE, with

only minor enhancements as yet, based on the availablity of greater computing

resources.

Below is a preliminary form of a document describing in detail the format and

syntax of the control file:

* Example of current syntax for fourfit control file  - last modified 94.1.16

* This file makes no semantic sense for a real experiment; rather, it is used

* to illustrate typical command syntaxes.

max_parity 0.001                       * global commands come first

ref_freq  8213.15

x_crc       discard

x_slip_sync keep

y_slip_sync discard

start -10

use_samples true

if station L and f_group X

freqs a+ b c d- e f g h

pc_phases abcdefgh 5 -11 12 38 -56 13.2 11 -29

pc_mode ap_by_ap

pc_freqs abcdefgh 10 10 1010 10 1010 10 1010 1010

if station L and f_group S

pc_phases ijkmn 4.5 -78 39 +12 0

pc_mode normal

if station A

pc_mode multitone

pc_period 30

pc_tonemask abcdefgh 0 0 8 0 4 0 5 0

if (station V or baseline KT) and source 3C279       * parentheses NYI

sb_win -0.5 0.5    mb_win 0.02 0.02  dr_win -1.0E-6 0.5E-6

else

sb_win 0.0 0.0     mb_win 0.02 0.02  dr_win -1.0E-6 0.5E-6

if scan 288-210210

sb_win .37 .37

if scan > 289-132510

skip true

if baseline K? and not scan 250-120000 to 251-235959

switched scan_start

period 30

gates abcfgh  0 30  0 10  15 25     0 10  15 25  0 30

* End of sample control file

SELECTOR KEYWORDS       VALUES

station            1 character

baseline           2 characters

source             string of 1-8 chars

f_group            1 character

scan               UT-epoch (special format), or:

< UT-epoch

> UT-epoch

UT-epoch1 to UT-epoch2  (inclusive time range)

SYNTACTIC KEYWORDS

if

else   (NYI)

and

or

not

()     (NYI)

<>

to

?

ACTION KEYWORDS         VALUES

max_parity         float

x_slip_sync        `keep', `discard', or an integer

y_slip_sync          "        "        "    "

x_crc              `keep' or `discard'

y_crc                "           "

freqs              n chars

index              n ints

pc_phases          n char string, followed by n floats

pc_freqs           n char string, followed by n floats

lsb_offset         float

ref_freq           float

pc_mode            `normal', `ap_by_ap', `manual', or 'multitone'

pc_period          int

pc_tonemask        n char string, followed by n floats

sb_win             2 floats

mb_win             "   "

dr_win             "   "

start              integer

stop                  "

skip               `true' or `false'

switched           `scan_start' or `each_minute'

period             int

gates              n char string, followed by 2n floats

adhoc_phase        'sinewave' or 'polynomial'

adhoc_period       float

adhoc_amp          float

adhoc_tref         float

adhoc_poly         <7 floats/integers (mixture OK)

ra_offset          float

dec_offset         float

use_samples        `true' or `false'

passband           2 floats

t_cohere           float

ionosphere         float

KEYWORD SEMANTICS

**scan selection**

skip          if this is set to true in the body of an if_block, then

any scans matching the if conditions will be skipped. Note: as of

99.2.19 fourfit will not properly skip data if f_group is specified.

**filtering**    determines whether or not each AP is accepted

max_parity    maximum allowable fraction of bytes in error

x_slip_sync   maximum # of frames w/ re-sync in reference station

y_slip_sync      "    "  "    "   "     "     " remote      "

x_crc         either keep or discard AP if reference time has CRC error

y_crc           "      "   "    "    "  "  remote      "   "   "    "

freqs         controls which frequency channels get included in the fit.

The letters a-p correspond to the order that the frequencies

appear in the root file. With no suffix, DSB is implied, if

both sidebands are present. A plus suffix denotes USB, a

minus is used for LSB.

index         alternate method used to select data channels, based on the

original corel index number. Not as well supported as freqs,

and is current being phased out.

start         start time for data to be included.

stop          stop    "   "    "  "   "    "

Arguments of start and stop are integers with an optional minus sign. A

positive integer is interpreted as an absolute time in seconds past the

hour (of the scan start time). When a minus sign precedes the start time

it is considered to be a time relative to, and later than, the

scheduled scan start. Similarly, a negative stop time precedes the

scheduled scan stop time, by the indicated number of seconds.

switched      turns on (frequency) switched mode, which discards some AP's

and keeps others, depending on a gating waveform

period        period in seconds of the gating waveform

gates         for each freq. channel, the starting delay and duration, in

seconds, of the gating waveform

passband      lower and upper bounds (in MHz) of the spectral passband of

data to be accepted, specified as RF frequencies.

**search**       control the fringe-searching process

sb_win        single band delay search window bounds, in us

mb_win        multiband     "     "      "       "     "  "; if the upper

bound (2nd number) is less than the lower bound (1st number),

then fourfit performs a "wrap-around" search, in order to handle

the case of a delay near to the multiband (semi-) ambiguity.

dr_win        delay_rate search window bounds, in us/s

ra_offset     apply right asc.offset (asec) to re-center search windows

dec_offset      "   declination  "   (asec) "      "        "      "

**corrections**  apply corrections to the data, either before or after fit

pc_mode       specify whether phase_cal mode is normal (model linear in

time is extracted from the data), manual (specified totally

by the user), ap_by_ap (phase cal is extracted independently

for each AP), or multitone (all tones in band are coherently fit,

and phase is extrapolated to the center of the band).

pc_period - in multitone mode (only), the phase can be estimated and applied

over each pc_period ap's, thus removing slopes or other drifts

in pcal (default is 9999)

pc_phases     phase_cal phases in deg, for each of the listed freq channels;

these offset phases are added to the underlying model, as

specified by pc_mode, above

pc_freqs      phase cal tone frequencies in KHz, for each of the listed

freq channels iff not in range -64..64. Inside of this range, the

value is interpreted as a tone #, with 1 being the 1st USB tone,

2 being the 2nd USB tone, etc. Negative tone #'s are used for

LSB tones.

pc_tonemask - in multitone mode (only):

The values for pc_tonemask form a bit-masked map of which

tones to *exclude* for this frequency channel. Thus 1

excludes the lowest tone, 2 the next lower tone, 4 the 3rd

lowest tone, etc. A value of 5, for example, would exclude

the lowest and the 3rd lowest tones (perhaps 10 KHz and 2.01 MHz).

lsb_offset    additive phase in degrees, for the LSB relative to the USB;

often necessary when correlating VLBA data against Mk 3

ref_freq      specifies a frequency in MHz at which the phase delay

is determined (default is total LO of first frequency)

adhoc_phase   specify mode of ad hoc phase corrections. No corrections

are made if this isn't present, or is set to false.

adhoc_period  For ad hoc sinewave model; the period in integer seconds.

adhoc_amp      "   "  "     "       "    amplitude in degrees of phase.

adhoc_tref    For both ad hoc phase models; the reference time in seconds

past the most recent hour.

adhoc_poly    For the ad hoc phase polynomial model; From 1-6 coefficients

describing a power-series model in time. (deg/sec^n)

use_samples   If true, use the sampler statistics (aka state counts) to

normalize the raw correlation sums to the equivalent analog correlation.

ionosphere    specified per station, in TEC (10^16/m^2) units

t_cohere      coherence time used in fringe fit (default is infinite)

SPECIAL KEYWORD VALUES

?         wild card character

keep      32767

discard       0

true          1

false         0

SPECIAL FORMATS

UT-epochs:

UT-epochs are expressed in the format ddd-hhmmss, where all 10

characters are necessary, including leading 0's if appropriate.

This format will match that of a scan directory, if the UT-epoch

that is being specified is an actual scan time.

GENERAL GUIDELINES

1) White space is ignored; i.e., multiple spaces and line feeds all

collapse to a single space.

2) Multiple commands per line are fine.

3) Comments: anything from an asterisk through the end of the line

is ignored.

4) Nested ifs are not allowed (or necessary). Nested parentheses in

an if condition are fine (NYI).

As of 94.1.16, parentheses are not supported. The logical operators,

in decreasing order of precedence are (not, and, or).

5) Wildcard "?" matches any single character for f_group, station, or

baseline, any string (of up to 8 characters) for source, and any

time-value for scan.

6) Phase cal is normally treated station by station.

7) Only freqs that are chosen for both stations in a baseline are

present in the fit.

8) If multiple if-blocks match a particular passes' choice of baseline,

f_group, source, and scan criteria, then the later values assigned

to each parameter overwrite the earlier ones.

The output of fourfit differs in some ways from FRNGE.  Below is a list of most

of the key differences in the fringe plot:

Principal differences between FRNGE plots and fourfit plots    CJL 4/11/94

-----------------------------------------------------------

1. Frequencies are labelled with alphabetic characters instead of numbers.

This scheme follows through to the channel-by-channel plots and information

on the lower half of the page.  These frequency identifiers are tied to

the LO frequencies, and are used to identify the sideband pairs in the

fourfit control file (keywords "freqs", "pcal_phases", etc.).  They are

ordered the same way as the LO frequencies in the type 2400 record of the root.

2. The L, A, U indicators for lower, dual and upper sideband data in the

segmented amplitude plots on the lower page are replaced by -, * and +.

This avoids possible confusion with the alphabetic phase indicators, with

better visual separation of amplitude and phase.

3. The T character in the all-channel segmented amplitude plot on the lower

page is replaced by 'A', to denote amplitude.  The 'P' remains, to denote

phase.

4. The phasecal phase plots on the lower page are keyed by the frequency code

letter.  Upper case means reference station, lower case means remote

station.

5. The FMGR file name on the 5th line of top and bottom pages is replaced by

the unix root extension code.

6. The start and stop times are calculated slightly differently.  There is no

important consequence of this change, but see item 9 below.

7. Counts of CRC errors, time discards, slip syncs and so on are slightly

different, due to the order in which operations are done in fourfit versus

FRNGE.  None of these differences are thought to be of any consequence.

In fourfit, some of the records are discarded early, before errors other

than bad time tags can be detected.

8. The MODE quantity (next to SNR on the upper page) has a completely different

meaning to the FRNGE number.  The pc mode indicators are a single character

for the reference station, followed by one for the remote station. The possible

characters are N (normal), A (ap_by_ap mode), and M (manual).

9. On the segmented plots on the lower page, the time span of the data is defined

to be the interval between the first and last APs with valid time tags.  This

definition differs slightly from FRNGE, so that the segmenting is somewhat

different.  In particular, fourfit will tend to have more segments with no

gap at the beginning or end, compared to FRNGE, on a dataset that starts

producing good data late (or stops early).

10. The fringe rate spectrum on the upper plot is gridded onto the page in a

slightly different fashion to FRNGE.  The changes are cosmetic only.  The logic

that dictates which points in other singleband delay boxes make it onto the

fringe rate plot also differs slightly from FRNGE.  The new algorithm is believed

to be more accurate.

11. The horizontal axes of the plots on both pages are not yet labelled.

12. When less than 14 channels are present, fourfit leaves blank space on the lower

page, below the segmented data plots.  FRNGE puts zeroes in this location.

13. Quality factors A, C and F are not supported yet (and may never be, depending

on postprocessing strategy decisions to be taken).

14. At the bottom of the lower page, fourfit places the names of the control file

used, and the full unix pathname of the output fringe file.  These replace the

HP-1000 specific equivalent information found at slightly different locations

on a FRNGE plot.

15. Fourfit does not separately list the manual phasecal values if used.  They can

be found from the MODE value (item 8 above) and the channel PCALPH1 and 2

numbers below the plot on the lower page.

16. When plotting the cross-power spectrum in the right-hand portion of the upper

plot, fourfit overwrites any existing characters (i.e. unlike FRNGE, the entire

cross-power spectrum is always visible).  Also, all phase points are plotted,

regardless of the amplitude at that point.  By default, cross-power spectra are

plotted when the scan SNR exceeds 100, rather than when the amplitude exceeds

1 percent as in FRNGE.  Forced plotting of the spectrum is available through

the -s option in fourfit.

17. OPT and SG in the extreme bottom right of the lower page are not filled in by

fourfit (they are specific to FRNGE special modes).

18. The special 'B' error code expanded fringe-rate plot is not supported by

fourfit.

19. The STAT field just above the SNR on the upper page is not filled in by fourfit.

There is a new field, "bc" just to the right of "me" on the right side of the

lower page.  This is a count of the number of AP's discarded due to a small

number of bits processed through the sine or cosine channels of the correlator,

normally a consequence of very low fringe rates (it stands for "bad count").

20. For the most part, FRNGE and fourfit results agree numerically to high accuracy.

Phases are normally within 0.1 degrees, and overall fringe amplitudes agree

to typically 0.1 percent.  Algorithmic differences may produce larger discrepancies

in cases dominated by noise.