Fringe Search

Algorithm and Implementation

For each sideband of each Accumulation Record :

1. Normalize correlation coefficients.

(10,000 = 100% correlated)

CosC(j) = c * (2*cosbits - #bits) / # bits

SinC(j) = dr_sign * c *(2*sinbits - #bits) / #bits

c =

j = correlation lag index (0 - 7)

# bits = total # of bits correlated in AP

dr_sign = sign of delay rate rotation

(special option - on lower sideband data, reverse sign of Sin correlation)

2.  FFT to Cross-Power spectrum

7

S(w) = SUM (CosC(j) + SinC(j)*i ) * exp(-2*PI*i*(j-4)*w/16)

j=0

w = video frequency index

16 point FFT, with correlation data sorted so that

delay offset = 0 is first.

3.  Adjust for Fractional Bitshift

S(w) = S(w) * exp(-2*PI*i*theta)

theta = (FB + dd) * (1 - int(SH)/SH ) * (w-4) / 16

FB = Fractional part of bitshift (type_corel.fbit)

SH = total bitshift * length of AP

dd = 0 if int(SH) is even

+.5 if int(SH) is odd and FB > 0

-.5 if int(SH) is odd and FB < 0

4. If phasecal is on this sideband:

CosPcal(st) = (2 * pcosbits - #pbits) / #pbits

SinPcal(st) = (2 * psinbits - #pbits) / #pbits

Pcal(st) = (CosPcal[st] + SinPcal[st] * i) * exp(-2 * PI *i* theta)

theta = 160 * s.p. * pcal_freq     		        (station 1)

= (160 + prt_offset + delayz) * s.p. * pcal_freq     (station 2)

s.p. = sample period

prt_offset = reference time offset from start time

delayz = correlator offset

For each Accumulation Period

1. Combine sidebands

SC[w] = S[w]     : upper sideband (w=1 to 7)

SC[32-w] = S[w]  : lower sideband (w=1 to 7)

SC[w] = 0        : w = 0 and w>7 and w<-7

SC = combined cross-power spectrum

2. FFT to Single Band Delay

7

D[sbd] = SUM SC[w] * exp(-2 * PI * i*w * (sbd-8) / 32) / 7.

w=-7

sbd= single band delay index (0 to 15)

32 point FFT, with single band data resorted so that sbd=8

is the central lag, and normalized by dividing by 7.

1. (special option - phasecal AP by AP)

D[sbd] = D[sbd] *  exp((pcal[fr,t,rem] - pcal[fr,t,ref]) - (avgpcal[fr,rem] - avgpcal[fr,ref]))

pcal = phase cal phase for freq fr at time t

avgpcal = phasecal for freq fr averaged over all times

For each delay channel :

For each frequency :

1. FFT across time to fringe rate spectrum

tmax

D[fr,L,sbd] = SUM D[t,sbd] * exp(-2 * pi * i * (L - Lsize/2) * t / Lsize)

t=0

fr= frequency index (0 to # of freqs)

L= fringe rate index (0 to Lsize -1)

t = time index

Lsize = size of FFT

= 4 * number of AP's but less than 1024

(special option - cut Lsize to 2 * num. of APs)

data is resorted after FFT so that L = Lsize/2 corresponds to 0 fringe rate

2. Grid fringe rate spectrum into delay rate spectrum

D[fr,dr,sbd] = ( D[fr,L,sbd]*(1 - (a-L)) + D[fr,L,sbd]*(a-L) ) * exp(-2*pi*i*theta) / Lsize

dr = delay rate index (0 to drsize-1)

drsize = size of delay rate spectrum

= 2 * number of AP's

a = fringe rate index corresponding to delay rate index dr

= ((dr-drsize/2) * (freq[fr]/freq[0]) * (drsize/Lsize) + Lsize/2)  mod Lsize

L = int(a)

theta = pcal_phase[reference] - pcal_phase[remote]

For each delay rate index

1. FFT across frequency to multiband delay spectrum

nfreq

D[mbd,dr,sbd] = SUM D[fr,dr,sbd] * exp (-2 * PI  * i * FS[fr] * (mbd - mbdsize/2) / mbdsize)

fr=0

mbd= multiband delay index

mbdsize = size of multiband delay spectrum

= # of points necessary to cover all frequencies (calculated in freq_spacing() )

FS[fr] = index in FFT for freq fr

= (freq[fr] - freq[0]) / largest common denominator in frequency spacings

2. Renormalize D[mbd,dr,d]

D[mbd,dr,d] = D[mbd,dr,d] / total # of Accumulation records

3. 3-dimensional search for mbd, dr, and d which give maximum value of D

Interpolation :

1. Interpolate to find phasecal rate by maximizing the amplitude of :

PC[pc_rate] = SUM SUM exp(theta)

fr=0 t=0

theta = pcal_phase[fr,t] - avg_pcal[fr] - 2 * PI * freq[fr] * pc_rate * A.P. * t

pcal_phase = phasecal phase for freq fr at time t

avg_pcal = phasecal phase for freq fr averaged over time

pc_rate = phasecal rate

A.P. = length of accumulation period

PC is evaluated at 3 equally spaced values of pc_rate, and a parabolic

interpolation is performed to find the peak.

This process is performed twice for each station, with the

pc_rate spacing = 1. and .5

Parabolic interpolation :

X = ( Y2 - Y0 ) / (2 * (2*Y1 - Y0 - Y2))

Y = Y1 + D*(Y2 - Y0)/4

2. Interpolate for precise values of residuals

Evaluate multiband delay function by direct summation

D[G,R,sbd] = SUM  SUM  D[fr,t,sbd] * VROT(t,R,G,fr,sb)

fr=0 t=0

VROT(t,R,G,fr,sb) = exp(-2 * PI * i * (

(freq[fr] * R * (A.P. * (t + .5) +E)) +

pc_phase[fr,rem] -pc_phase[fr,ref] +

(freq[fr] - freq[0]) * G +

(sbd' - sbd) * sb ))

R = Delay rate (us /s)

E = Offset of reference time from start of data

G = Group delay (us)

sbd' = interpolated single band delay index

sb = +1 upper sideband  -1 lower sideband 0 both

D is evaluated at three points centered on the current best estimate.

A parabolic interpolation of the magnitude of D to improve these estimates.

Interpolation is peformed first for delay rate, then group delay, then singleband delay.

The process is repeated three times, with the spacing halved on the third iteration.

3. Corrections

a) amplitude correction due to fringe rate

D_max = D_max * theta / sin(theta)

theta = R * freq[0] * A.P. * PI

b) if pcal not AP by AP, adjust for  phasecal rate

R = R + (pc_rate[rem] - pc_rate[ref])

IMPLEMENTATION

The actual fringe searching is controlled by the search() subroutine

which is called from fourfit(). Search initializes several settings, then processes

through the data calling upon other subroutines to do major computations.

Search() summary :

------------------

Initialize variables;

for (fr=0;fr < number of frequencies; frequency ++)

for (ap=0; ap< number of aps ap ++)

norm();

rotate_pcal();

freq_spacing();

search_windows();

for (lag =0; lag < 15 ; lag ++)

{

for (fr=0 ; fr < number of frequencies; fr ++)

delay_rate();

for (dr = 0; dr < dr spectrum size ; dr ++)  /* dr = delay rate */

{

fill data array;

FFT();   /* to multiband spectrum */

for (mbd =0 ; mbd < mbd spectrum size ; mbd ++)

find maximum & update();

}

}

interp();

Catalog of other subroutines :

-----------------------------

FFT1()  -- FFT based on FOUR1 in Numerical Recipes

norm() -- performs calculations required for every accumulation record.

calculates everything up to single band delay function.

Norm() summary :

for (sideband=0; sideband<2 ; sideband++)

{

normalize correlations;

FFT to XPower spectrum;

Correct for fractional bitshift;

extract phasecal;

}

combine sidebands;

FFT to singleband delay;

rotate_pcal() -- counter-rotates for phasecal if desired.

freq_spacing() -- calculates spacing of frequencies in FFT to multiband delay

search_windows() -- determines size of spectra and search boundaries;

delay_rate() -- calculates fringe rate spectrum and grids it into delay rate spectrum

update() -- calculates residual values from delay rate, sbd and mbd indices

interp() -- interpolates phasecal rate and residuals & performs several corrections to results

Interp() summary :

for (interp=1; interp<=2; interp++)

{

for(index=1;index<=3;index ++)  /* evaluate PC at three points */

PC[index] = sum over time & freq of exp(-2 *pi *i *theta);

parabola(PC[3]);

}

for (interp=1; interp<=3; interp ++)

for (var=1;var;<=3;var++)  /* interpolate for 3 variables */

{

for(index=1;index<=3;index ++)

{

for(fr=0;fr<number of frequencies; fr++)

for(ap=0;ap<number of ap's ; ap++)

D[index] += D[fr,ap,sbd] * VROT();

}

parabola(D[3]);

update residuals;

}

amplitude & delay rate corrections;

vrot() -- calculates complex rotator used in evaluating delay resolution function

parabola() -- performs parabolic interpolation of 3 equally spaced points, returning the peak

Getting data in and out of Search()

----------------------------------

Search has two main data parameter structures - param and status. Param consists of values which

should be determined before search is called, such as special options & restrictions and parameters

such as sample rate or number of frequencies to be processed. Status contains the values which

search generates itself and needs to remember. Included in status are values which must be printed

on the fringe output but are not contained in the type fringe record.

Corelator data is passed to search via the corel[] structure. Corel[] is an array of freq_corel structures,

each corresponding to one frequency. Each freq_corel structure contains information specific to that frequency,

as well as an array of data_corel accumlation records. Each accumulation record contains  a pointer to

a Type 51 record for each sideband, as well as arrays to hold the singleband delay spectrum and extracted

phasecal as these are calculated. Presence of data in all of these structures is indicated by a non-null

pointer.

Search examines all data that is passed to it. Therefore selection of certain frequencies

or time ranges should be done by not placing these records in the data structure. Individual accumulation

records can be ignored by setting their pointer null.

Parameter structures

--------------------

struct type_param {

double 		cor_limit;     	/* Maximum correlation amplitude */

int		corel_offset;   /* Corelator offset (bits)	*/

int		nfreq;	       	/* Number of frequency channels   */

double		acc_period;   	/* Length of accumulation period (sec)  */

int		num_ap;		/* Number of Accumulation Periods */

double		samp_period;	/* Length of sample period	*/

short		pc_ap;	       	/* Use pcal per AP = 1 		*/

short		lsb_reverse;   	/* Reverse lower sideband = 1 */

short		pulsar;		/* Pulsar mode = 1 */

short 		man_pcal;	/* Manual phasecal = 1 */

short		fast_dr;	/* Reduce size of fringe rate FFT =1 */

double 		prt_offset;     /* offset from reference time */

double		win_sb[2];	/* Single band search window microsec */

double 		win_dr[2];	/* Delay rate search window microsec per sec */

double 		win_mb[2];	/* Multi band search window  microsec */

double 		inv_sigma;	/* 1 / (1 sigma noise) */

double		LO2_hi_phs;	/* LO2 HI phase */

double		LO2_hi_freq;	/* LO2 HI freq */

double		start;		/* start time */

double		stop;		/* stop time */

double		epoch_time;	/* Epoch time */

double		ref_st_epoch;   /* Ref. station epoch  ms */

};

struct type_status {

double		freq_space;		/* Freq spacing in FFT to MBdelay */

int		grid_points;		/* # of points in FFT to MBdelay */

double		freq_spread;		/* Sum of (Freq - avg freq) squared */

int		large_errors;  		/* # of large errrors */

long		total_ap;		/* Total number of ap records processed (both sb's )*/

int		ap_num[2][MAXFREQ];	/* # of aps for each sideband & freq */

double		epoch_err[MAXFREQ];     /* Epoch error  (offset from reference time)  */

double		epoch_off_cent;  	/* Epoch offset from center  */

int		mb_index[MAXFREQ];	/* index for freq in FFT to MBdelay */

double		pc_phase[MAXFREQ][2];  	/* phase-cal phase */

double		pc_amp[MAXFREQ][2];    	/* phase-cal magnitude */

double		pcals_accum;	  	/* # of phase-cals for current channel */

int		space_err;		/* =1 if error if freq spacing is too large */

int		drsp_size;		/* # of points in delay rate spectrum   */

int		f_rate_size;		/* # of points in FFT to Fringe rate */

int		freq;			/* Current frequency  */

int		lag;			/* Current lag #  */

int		dr;			/* Current delay rate #  */

int		mbd;			/* Current multi band delay #  */

int		ap;			/* Current acc per */

double		dr_max;			/* Delay rate at max */

double		corr_dr_max;		/* Delay rate max corrected for pcal rate */

double 		sbd_max;		/* single band delay at max*/

double		mbd_max;		/* multi band delay at max */

int		max_delchan;		/* Single band delay channel at max */

double		delres_max;		/* max value of delay resolution function */

double		rate_sep;		/* Rate separation of FFT points for freq #1 */

double		mbd_sep;		/* Multi band delay separation of FFt points */

double		sbd_sep;		/* Single band delay separation  microsec */

double		search_amp;		/* Cor. amplitude after initial search */

double		interp_amp;		/* Amplitude after 1st interpolation */

double		pc_rate[2];    		/* Interpolated phasecal rate */

double		int_pc_amp[2];          /* Interpolated phasecal amplitude */

double		amp_corr_fact;		/* Amplitude correction factor for interpolation */

double		amp_rate_corr;		/* Rate correction to amplitude */

int		interp_err;		/* =1 if error in interpolation */

double		AN;			/* Normalization from FFT (from FRNGE) */

int		win_sb[2];		/* Single band search indices */

int 		win_dr[2];		/* Delay rate search indices */

int 		win_mb[2];		/* Multi band search indices */

int		pts_searched;		/* Number of points searched */

complex		fringe[MAXFREQ+1];	/* Fringe phase & amp for each freq */

double		sbdbox[MAXFREQ+1];	/* Single band delay box for each freq */

double		snr;			/* Signal to Noise ratio */

double		false_det;		/* Probability of false detection */

double		th_timerms_phase;	/*  theoretical RMS phase with time */

double		th_timerms_amp;		/* theoretical RMS amp with time */

double		th_freqrms_phase;	/*  theoretical RMS phase with freq */

double		th_freqrms_amp;		/* theoretical RMS amp with freq */

double		timerms_phase;		/* RMS phase with time */

double		timerms_amp;		/* RMS amp with time */

double		freqrms_phase;		/* RMS phase with freq */

double		freqrms_amp;		/* RMS amp with freq */

double		integ_time;		/* Total integration time */

double		inc_avg_phase;		/* Fringe phase for incoh. avg. over freq*/

double 		inc_avg_amp;		/* Fringe amp. from inc. sum   of all points */

double		coh_avg_phase;		/* Fringe phase from coh. sum over freq */

double		inc_avg_amp_freq;	/* Fringe amp from incoh sum over freq */

double		GHA;			/* Greenwich hour angle (hrs) */

double		apdelay;		/* Adjusted apriori delay */

double		amp_err;		/* Error in fringe amplitude */

double		apphase;		/* Apriori phase */

double		resid_phase;		/* Residual phase */

double		mod_resid_phase;	/* Modified residual phase */

double		phase_err;		/* Error in fringe phase */

double		resid_ph_delay;		/* Residual phase delay */

double 		ph_delay_err;		/* Error in phase delay */

double		rate_ra_width;   	/* Delay rate width (RA) */

double		rate_dec_width;		/* Delay rate width (dec) */

double		sbd_ra_width;		/* Single band delay width (RA) */

double		sbd_dec_width;		/* Single band delay width (dec) */

};

Data structures

---------------

extern struct freq_corel corel[MAXFREQ]

struct freq_corel {

double frequency;

int sband;

int pcal_location;

double pcal_freq;

struct data_corel *data;

};

struct data_corel {

int flag;

struct type_corel *apdata[2];

complex sbdelay[16];

complex phasecal[2];

};

union pos7_8 {

long		fphase;			/* MkIII rotator phase end of AP */

short		frame_cnt[2];		/* MkIIIA #x/y discards/resyncs */

};

struct cor {

long		coscor;			/* Cosine correlation */

long		sincor;			/* Sine correlation */

};

struct pcals {

long		cos;			/* pcal cosine correlation */

long		sin;			/* pcal sine correlation */

long		bits;			/* pcal bits correlated */

long		err;			/* # bytes with parity error */

};

struct type_corel {	/* NOTE: this is 128 bytes, half a record */

unsigned short	base_index;		/* baseline # and index # encoded */

short		flags;			/* lots of flag bits */

short		status;			/* General status */

unsigned short	bitshift;		/* Bit shift rate (absolute) */

long		time;			/* Time at end of last PP in AP */

union pos7_8	x;			/* Fphase or frame_cnt */

long		delayz;			/* Delay (bits) */

float 		fbit;			/* fractional bitshift center of AP */

struct cor	xcor[8];		/* actual correlation results */

long		cosbits;		/* cosine bits correlated */

long		sinbits;		/* sine bits correlated */

struct pcals	xypcals[2];		/* Pcal info, X then Y */

};