Documentation for the program "pratio" CJL, 29 October 1997

-----------------------------------------------------------

COMMAND NAME:   pratio

SYNOPSIS:  Computes scan-averaged ratios of cross and parallel hands

SYNTAX:  pratio [-m msglevel] filename

OPTION FLAGS:	-m msglevel

Verbosity control parameter, ranging from -3 to +3,

with lower numbers being more verbose.  The default

value is 1.

ARGUMENTS:	filename

This argument specifies the input dataset.  The name

given is the root name of a set of up to four A-files,

which contain scan-summaries of the four polarization

correlation products.  The program will look for the

four files "filename.xx", where "xx" is "RR", "LL",

"RL" and "LR".  One or both of the parallel hand files

should have the coherence time fields filled in, which

will be propagated to the other polarizations.  The

program cannot check that the contents of the four

files actually are the stated polarizations, so the

input files must be carefully prepared.

ENVIRONMENT:	None

DESCRIPTION:

This program is responsible for forming various polarization ratios for

polarization-sensitive mm-VLBI datasets.  The sequence of operations that

must be performed is (1) segment each correlation product to the same

coherence time, (2) form rl/ll, lr/ll, rl/rr, lr/rr, rr/ll ratios for each

segment, (3) compute vector scan averages for each ratio, and (4) write the

results out into a new A-file.  In order to process large datasets, it is

necessary to do a lot of sorting, to match up scans and segments, so pratio

makes multiple external calls to aedit, and keeps scratch files around on

disk.  It also must do the segmenting, which is handed off to fringex.  A

modest amount of computation is done internally, but the main function of

pratio is to automate a highly repetitive process that would be very

time-consuming to do by hand, or via shell scripts constructed on an

experiment by experiment basis.

In principle, the program should handle missing data in different

polarizations, and multiple experiment numbers, gracefully.

There is a lot of computation involved, and considerable file IO which could

be eliminated at the cost of increased complexity.  However, execution

times are not considered to be a major issue.