Documentation for the program "Coterp", SSD, 11/06/96

----------------------------------------------------

COMMAND NAME:   Coterp

SYNOPSIS: 	Determines coherence times to be used for coherently undetected

scans.  Uses least squares fitting and interpolation on coherence

times found for firm detections.

SYNTAX:  coterp [-x] [-d device] [-o outfile] [-m msglevel] fitlist masterlist

-x	open x-window for graphics display

-d 	select graphics device

-o 	specify output file

-m	set message level

OPTION FLAGS:	-d device  [Not yet implemented]

Specifies the output graphics device for the plots

of station coherence time.

The specification is in the standard PGPLOT form

of filename/devicetype (e.g. test.ps/ps)

-m msglevel

Verbosity control parameter, ranging from -3 to +3,

with lower numbers being more verbose.  The default

value is 1.

-o outfile

If specified, this flag directs the A-file output

of cofit to the named file.  If omitted, coterp

writes its output to stdout.

-x

Shorthand for "-d /xw", the xwindow display.

ARGUMENTS:	fitlist

A-list of coherence fitted scans as output by cofit.  Scans

in this file are assumed to have already been processed by

cofit which will have filled in no-loss and search

coherence time fields.  These scans should be firm coherent

detections since they will be used to find coherence times

for non-detection scans.

masterlist

An A-list of all scans to be processed which includes not

only the detections but also non-detections.  The scans

originally processed by cofit to produce the fitlist are

included in masterlist.

ENVIRONMENT:	None.

DESCRIPTION:	For purposes of fringe searching and for evaluating visibility

amplitude, one needs to know the coherence time of the data.  The

program COFIT determines these times for scans which have already

been detected by coherent averaging methods (such as the program

FOURFIT).  COTERP uses the values determined in COFIT to interpolate

coherence times on baselines for which fringes are not yet detected.

To do this, COTERP first uses a least squares algorithm to 'stationize'

the baseline based coherence times from COFIT.  For a given scan time,

this results in a coherence time for each station which is descriptive

of the atmospheric turbulence above a given station at a given

elevation.  The algorithm to stationize the coherence times makes

the assumption that the inverse square of a baseline coherence time

is equal to the sum of inverse squares of the station coherence

times.

Each station coherence time is then adjusted using a theoretical

elevation dependence to take into account the coherence change

with time.  Once this is done, the adjusted station coherence

times are interpolated for the unfitted scans in the masterlist.

The final step inverts the coherence time relation above to

calculate baseline coherence times from the interpolated station

based times.  All scans in the resulting output file will have

either coherence times determined by COFIT (previous detections)

or coherence times interpolated by COTERP (previous non-detections).